# Smart Thermostat Prototype Report

## Introduction

SysTec’s initiative to enter the smart thermostat market requires a prototype that demonstrates core embedded system functionality while aligning with business requirements. The purpose of this project was to design and implement a working thermostat using a Raspberry Pi development board. The system integrates multiple hardware peripherals, including a temperature sensor, liquid crystal display (LCD), light-emitting diodes (LEDs), and buttons, to simulate real-world thermostat behavior. In addition to hardware control, the system also outputs data through a serial interface to represent communication with a remote server.

## Support for Hardware Peripherals

The thermostat prototype successfully supports all required peripherals through coordinated software control. The temperature sensor, connected via the Inter-Integrated Circuit (I2C) bus, provides real-time environmental data. The system reads temperature values from the AHT20 sensor and converts them into Fahrenheit for internal decision-making and display.

General-purpose input/output (GPIO) pins are used to manage both input and output devices. Three buttons are configured using the GPIOZero library to detect user input through interrupt-driven events. These buttons allow the user to cycle between thermostat states (off, heating, cooling) and adjust the temperature set point.

Two LEDs, connected through GPIO pins and controlled using pulse-width modulation (PWM), visually represent the system’s behavior. In heating mode, the red LED pulses when the current temperature is below the set point and remains solid when the target temperature is reached. In cooling mode, the blue LED pulses when the temperature exceeds the set point and remains solid when the system stabilizes. This behavior provides immediate visual feedback to the user.

The LCD display is used to present system information. The first line continuously shows the current date and time, while the second line alternates between the current temperature and the system state with the temperature set point. This ensures that the user can easily monitor both environmental conditions and system status.

## Cloud Connectivity via Wi-Fi

Although the prototype uses a serial interface (Universal Asynchronous Receiver-Transmitter, UART) to simulate communication, the final system design will require Wi-Fi connectivity to transmit data to SysTec’s cloud-based analytics platform. The Raspberry Pi supports Wi-Fi natively, making it suitable for early-stage development and testing.

In a production environment, the thermostat would transmit structured data packets over Wi-Fi using protocols such as Hypertext Transfer Protocol Secure (HTTPS) or Message Queuing Telemetry Transport (MQTT). These protocols allow efficient, secure communication between the device and cloud services. The system would periodically send updates containing the thermostat state, current temperature, and set point, enabling remote monitoring and data analysis.

## Hardware Architecture Comparison

Three hardware architectures were considered for the production version of the thermostat: Raspberry Pi, Microchip microcontrollers, and Freescale (NXP) microcontrollers.

The Raspberry Pi provides a full operating system, built-in Wi-Fi, and significant processing power. It supports all required peripherals and simplifies development due to its high-level programming environment. However, it consumes more power and may be more expensive than necessary for a dedicated thermostat device.

Microchip microcontrollers offer a more lightweight and cost-effective solution. These devices are designed for embedded applications and can support GPIO, I2C, and UART interfaces. However, they often require additional components or modules to enable Wi-Fi connectivity, which increases design complexity.

Freescale (NXP) microcontrollers provide a balanced solution with strong support for embedded systems, efficient power usage, and compatibility with integrated wireless modules. These devices are widely used in industrial applications and offer sufficient memory and processing capabilities to support thermostat functionality.

## Recommended Architecture

Based on the project requirements, a Freescale (NXP) microcontroller with integrated or easily supported Wi-Fi capability is the most appropriate choice for production. This architecture meets all key business requirements: it supports the necessary peripherals, provides efficient wireless communication, and includes sufficient flash memory and random-access memory (RAM) to handle the thermostat’s logic.

While the Raspberry Pi is ideal for prototyping due to its flexibility, a microcontroller-based solution is better suited for a commercial product because of its lower power consumption, reduced cost, and more streamlined hardware design.

## Conclusion

The thermostat prototype demonstrates a successful integration of hardware and software components to meet SysTec’s initial requirements. The system accurately reads temperature data, responds to user input, controls output devices, and simulates server communication. By extending this design with a Wi-Fi-enabled microcontroller architecture, SysTec can develop a scalable and efficient smart thermostat solution capable of competing in the growing smart home market.